LANDSAT Application and Classification Disaster

Area on Yom River  Basin

Vicharn Amarakul1

Director of  Regional Geo-Informatics and Space Technology,

Lower Northern Region, Naresuan University, Muang District, Phisanulok, Thailand

v_amarakul@yahoo.com

Sirirat Sanyong2

Lecturer, Faculty of Agriculture, Natural Resources and Environment

Naresuan University, Muang District, Phisanulok, Thailand

siriratsy@hotmail.com

Abstract

The area of Yom River Basin had  23616 square kilometer which covered 11 provinces consisted of Phayao, Nan, Lumpang, Phrae, Utradit, Tak, Sukhothai, Phitsanulok, Kamphaengphet, Phichit and Nakornsawan provinces. There are eleven sub-basin in Yom River Basin and some areas in this basin was in serious flood in rainy season. LANDSAT images was used for classification the land use, calculation and estimation the direction of water flow and the severity of flooding. The data base for estimation flooding severity or the probability of flooding in each sub-basin were consisted of annual rainfall, slope, the boundary from the main rivers, the density of rivers and canals, area of sub-basin, land covered and  soil textures. The result from this study was showed that the highest risk flooding area in Yom River Basin must be occurred on Phitsanulok. The moderate risk had classified in the area of Kampangphet, Sukothai, and Phichit provinces, respectively.  The highest sensitive areas on flooding had covered 11,658.22 square kilometer or 60.99% of Yom River Basin. The high, moderate and mildly sensitive areas had covered 3,850.33 ; 1,582.34  and 2,025.11 square kilometer or 20.14%  8.28% and  10.59%, respectively.

Keyword

Yom River Basin, Phayao, Inundate, Nan, Lumpang, Phrae, Utradit, Tak, Sukhothai, Phitsanulok, Kamphaengphet, Phichit, Nakornsawan, Thailand

Introduction

Meteorological condition in Yom river basin is under southwest and northeast monsoons.

For the whole basin, the average annual rainfall, temperature and relative humidity were 1,118 mm, 26.4 o C and 72%, respectively. The maximum rainfall is in August and September. Rainfall is very important weighting factor.  Flooding situation in lower northern and middle region of Thailand had caused by over water flow from the northern region. In 2002, Strom had attacked the northern region for many times. Therefore, the disastrous consequence was heavy flooding in lower northern region, 593,118 persons were in trouble, agriculture lands were destroyed 453,101.3 ha. The infectious diseases were epidemic by water. From this reason, the Regional Center of Geo-informatics and Space Technology, lower Northern Region, Naresuan University, had very interesting and concentrate on monitoring the disaster which effecting from flooding.

Objectives

- To draw the boundary of the risk area on flooding in Yom river basin.

- To monitor the flooding direction and severity in the difference height on land

Study area

The study site in Yom river basin is covered 11 provinces, there are Phayao, Nan. Lumpang, Phrae, Uttradit, Tak, Sukothai, Phitsanulok, Kamphaengphet, Phichit and Nakornsawan provinces.

Material

For this task, the area where flooding happening was interpreted from LANDSAT imagery.

# Methodology

- Study on the correlation between topography and possibility to flooding

- Analyze opportunity for the occurrence of the flood inundate was done by overlay and weighting

variation data such as rainfall data, terrain mapping unit, main rivers boundary, density and size of water body boundaries in sub-basin, agricultural land use/cover and soil texture (drainage) as table 1 and 2.

- Depicting flood inundation maps at 1:50,000 scale in difference inundation levels and showed the

risking area in each districts which located on Yom river basin.

Density of water bodies in sub-basin

Density of water bodies in sub-basin had direct correlated with drainage, in case of high density of water bodies would be correlated with good drainage. The density of water bodies was calculated by the sum of whole of length of water bodies in each sub-basin divided by size of   sub-basin

Size of sub-basin

Sub-basin size could classified the inundate opportunity. The inundate criteria was defined by sub-basin size, the area more than 2,601 km2  defines as high opportunity to be inundation and the area less than 1,000 km2  defines as no opportunity to be inundation.

Agricultural land use/cover

Yom river basin had a complex topographies, it was hilly in the upper parts and flood plain in the lower parts. Agricultural land use/covers were sorghums, maize, paddy fields, sugarcanes, orchards, forestry and deforest area. The highest risk of inundate for agricultural land use/cover was paddy field.

Soil texture (Drainage)

Soil texture was correlated with drainage. The highest risk of inundate would occurred in the worst drainage soil. In controversy, good drainage would be the least risk of inundate.

Table 1	Grading/Weight values for flooding severity which base on inundate factors.

| Items | Severity or Weight value of inundate factors |
| --- | --- |
| 1. Annual rainfall | 7 |
| 2. Boundary from main rivers | 6 |
| 3. Slope | 5 |
| 4. Density of water bodies in sub-basin | 4 |
| 5. Size of sub-basins | 3 |
| 6. Agricultural land use/cover | 2 |
| 7. Soil Texture (Drainage) | 1 |

Table 2	   Factors weighting varies according to type of data

| Factors Variation | Class | Weighting |  |
| --- | --- | --- | --- |
|  |  | factors | Severity 1/ |
| 1. Annual rainfall | > 1,200 mm. 1,100 - 1,200 mm. 1,000 - 1,100 mm.. < 1,000 mm. | 7 | 4 3 2 1 |
| 2. Boundary from main rivers | 0 – 0.5 km. 0.5 – 1.0 km. 1.0 – 5.0 km. > 5.0 km. | 6 | 4 3 2 1 |
| 3. Slope | 0 – 5% 6 –10% 11 – 15% > 15% | 5 | 4 3 2 1 |
| 4. density of water bodies in sub-basin | 0.1 – 0.35 km./sq.km. 0.36 – 0.71 km./sq.km. 0.71 – 1.00 km./sq.km. > 1.00 km./sq.km. | 4 | 4 3 2 1 |
| 5. Size of sub-basins | > 2,601 sq.km. 1,801 – 2,600 sq.km. 1,001 – 1,800 sq.km. < 1,000 sq.km. | 3 | 4 3 2 1 |
| 6. Agricultural land use/cover | Paddy field Field crops Orchards and trees Forests | 2 | 4 3 2 1 |
| 7. Soil Texture (Drainage) | Worse Bad Moderate Good | 1 | 4 3 2 1 |

Remarks :  1/  1 = Low/Non	2 = moderately		3 = high		4 = very high

Inundate Equation

x=i=17wixiwi

x    = average of risk on inundate area

xi   = average of risk in each class

wi  = weight value of each class

i    = class priority from 1, 2, 3, …, 7

Percentage of Inundate Severity Area = 7Ra+6Di+5Sl+4De+3Si+2Ls+1So7+6+5+4+3+2+1×1004

7Ra	  = Weight value of risk area analyze by average of annual rainfall

6Di   = Weight value of risk area analyze by boundaries from main rivers

5Sl   = Weight value of risk area analyze by from slope

4De	  = 		Weight value of risk area analyze by from Density of water bodies in sub-basin

3Si    = Weight value of risk area analyze by size of sub-basin

2Ls  =   Weight value of risk area analyze by agricultural land use/cover

1So   =   Weight value of risk area analyze by soil texture (drainage)

Risk severity had been grading as following

0 – 50%        low severity / no severity

51 – 60%       moderately severity

61 – 70%       high severity

>70%         very high severity

Result

Inundate Analyze by rainfall

Inundate risk in Yom (Fig. 3) River Basin had been analyzed by rainfall found that the high risk area was only 2.33 percentage or 505.47 km2 which area belong to Phayao (in Chiang Khum, Chiang Muan and Pong districts), Nan

(in Tha Wang Pha, Ban Luang and  Song Khwae districts), Phrae (in Wang Chin and Den Chai districts) , Uttradit

(in Laplae district)  and Sukhothai  (Si Satchanalai) provinces. The low or non inundate area  was in Lampang, Phrae, Sukhothai (Thung Saliam and Sawankhalok district), Kamphaengphet (Tung Sai, Sai Ngam and Bueng Samakkhi )and Phichit (Sam Ngam, Pho Prathap Chang and Pho Thale ) Provinces.

Inundate Analyze by boundaries from the main river

The highest inundate risk area was 3952.94 km2 which belong to Phrae (Wang Chin district), Sukhothai

(Si Satchanalai, Si Samrong and Mueang Sukhothai districts), Phitsanulok ( Bang Rakam  district) Kamphaengphet (Sai Ngam districts) and Phitchit (Sam Ngam and  Pho Prathap Chang districts) .

Inundate Analyze by Slope

Their slope was correlated with the runoff velocity , the provinces was very highly opportunity to be inundated were Phrae, Sukhothai, Uttradit, Phitsanulok, Kamphaengphet, Phichit, and some parts of Nakornsawan Province.  It could be calculated as 16,123.19 km2 (Fig.4 ).

Inundate Analyze by Density of water bodies in sub-basin

High density of water bodies in sub-basin was higher condition for drainage the over flow than the low density of water bodies.  This study identified the density of water bodies as the total length of all water bodies divided by size of basin. From this analysis, it showed that the highest inundate risk area  was in Phitsanulok, Kamphaengphet and Phichit provinces which covered 2,872.46 km2(Fig.5 ).

Inundate Analyze by Size of Sub-basins

The size of sub-basins was bigger than 2,601 km2, had been shown as very highly inundate risk area. In controversy, the size of sub-basin was smaller than 1,000 km2, had been shown as non inundate risk area. Phrae, Sukhothai, Phitsanulok, Kamphaengphet, Phichit and some parts of Nakornsawan provinces were shown as the very

highly risk in inundate, which covered 8,350.33 km2 (Fig.5 ).

Inundate Analyze by agricultural land use/cover

Paddy field area was weighted as the very highly inundate risk area, which covered 12,567.84 km2. The inundate area covered Sukhothai, Phitsanulok, Kamphaengphet, Phichit and some parts of Nakornsawan provinces(Fig.2 ).

Inundate Analyze by Soil Texture (Drainage)

Slow drainage was weighted as the high inundate risk area which covered 118.9 km2  and was in Phayao (Chun and Pong districts ), Sukhothai (  Si Satchanalai and Ban Dan Lan Hoi districts) Phitsanulok (Bang Rakam district), Kamphaengphet (  Mueang Kamphaeng Phet and Sai Ngam districts) and Phichit (Sam Ngam and Pho Thale districts) (Fig.4 ). All size and percentage of inundate risk area in Yom River Basin had been shown as Table 3 and 4.

Percentage of inundate severity analyzed by the provinces which located on Yom river basin had showed that Phrae and Sukhothai provinces were 100 percentage risk on inundate. The most area in Phrae province about 3,635.44 km2 had effected on inundate from moderately to high severity or 45 percentage of  Phrae province. Also the most area in Sukhothai province about 4,358.51 km2 had effected on inundate from moderately to high severity or 55.08 percentage of Sukhothai province. Analyze the risk area base on all factors had showed that the area which belong to Nakornsawan, Uttradit, Tak and Nan provinces were only 0.54, 1.35, 3.24 and 11.18 of their provinces, respectively, risk on inundate. (Fig.6) and (Table 6 and 7 )

Table 3	Analyze the size of inundate risk areas in Yom River Basin

| Factors | Size of Inundate Risk Area (km2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Low / Non | Moderately | High | Very High | Total |
| Annual Rainfall | 5,893.22 | 10,146.36 | 7,025.95 | 505.47 | 23,616.00 |
| Boundaries from main river | 5,355.93 | 11,130.49 | 3,176.64 | 3,952.94 | 23,616.00 |
| Slope | 4,774.53 | 955.62 | 1,762.65 | 16,123.19 | 23,616.00 |
| Density of water bodies in sub-basin | 0.00 | 15,456.74 | 5,286.80 | 2,872.46 | 23,616.00 |
| Size of sub-basin | 2,025.11 | 1,582.34 | 11,658.22 | 8,350.33 | 23,616.00 |
| agricultural land use/cover | 8,777.57 | 1,732.32 | 538.27 | 12,567.84 | 23,616.00 |
| soil texture (drainage) | 18,711.91 | 306.18 | 4,479.39 | 118.53 | 23,616.00 |

Table 4	Analyze the percentage of inundate risk areas in Yom River Basin

| Factors | Percentage of Inundate Severity in Risk Area |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Low / Non | Moderately | High | Very High | Total |
| Annual Rainfall | 24.96 | 42.96 | 29.75 | 2.33 | 100.00 |
| Boundaries from main river | 22.68 | 47.13 | 13.45 | 16.74 | 100.00 |
| Slope | 20.22 | 4.05 | 7.46 | 68.27 | 100.00 |
| Density of water bodies in sub-basin | 0.00 | 65.45 | 22.39 | 12.16 | 100.00 |
| Size of sub-basin | 8.58 | 6.7 | 49.36 | 35.36 | 100.00 |
| agricultural land use/cover | 37.17 | 7.34 | 2.28 | 53.22 | 100.00 |
| soil texture (drainage) | 79.23 | 1.3 | 18.97 | 0.50 | 100.00 |

Table 5	Percentage of Inundate Severity in Risk Area on provinces in Yom river basin

| Provinces | Percentage of Inundate Severity in Risk Area |  |  |  |
| --- | --- | --- | --- | --- |
|  | Low/non | moderately | # high | Very high |
| Kamphaengphet | 0.53 | 8.81 | 17.28 | 14.6 |
| Tak | 1.82 | 1.42 | 0.00 | 0.00 |
| Nakornsawan | 0.00 | 0.09 | 0.3 | 0.15 |
| Nan | 7.56 | 2.81 | 0.7 | 0.11 |
| Phayao | 11.43 | 13.25 | 7.98 | 4.78 |
| Phichit | 0.02 | 7.2 | 10.05 | 8.81 |
| Phitsanulok | 0.00 | 0.37 | 8.89 | 22.55 |
| Phrae | 37.01 | 24.42 | 21.44 | 11.52 |
| Lumpang | 24.9 | 13.78 | 5.15 | 2.05 |
| Sukhothai | 16.39 | 27 | 28.08 | 35.4 |
| Uttradit | 0.34 | 0.85 | 0.13 | 0.03 |
| Total | 100.00 | 100.00 | 100.00 | 100.00 |

Table 6     Size of Inundate Risk Areas on provinces in Yom river basin

| Provinces | Size of Inundate Risk Areas (km2) |  |  |  | Total |
| --- | --- | --- | --- | --- | --- |
|  | Low/non | moderately | # high | Very high |  |
| Kamphaengphet | 25.37 | 711.21 | 1341.57 | 439.47 | 2517.62 |
| Tak | 87.10 | 114.49 | 0.00 | 0.00 | 201.59 |
| Nakornsawan | 0.00 | 7.61 | 22.91 | 4.66 | 35.18 |
| Nan | 360.72 | 226.48 | 54.45 | 3.28 | 644.93 |
| Phayao | 545.48 | 1069.09 | 619.34 | 144.02 | 2377.93 |
| Phichit | 1.11 | 580.63 | 779.95 | 265.21 | 1626.90 |
| Phitsanulok | 0.01 | 29.82 | 689.58 | 678.91 | 1398.32 |
| Phrae | 1766.52 | 1971.00 | 1664.44 | 346.74 | 5748.70 |
| Lumpang | 1188.52 | 1112.10 | 399.59 | 61.81 | 2762.03 |
| Sukhothai | 782.11 | 2178.81 | 2179.70 | 1066.09 | 6206.72 |
| Uttradit | 16.21 | 68.83 | 10.05 | 0.98 | 96.07 |
| Total | 4773.15 | 8070.07 | 7761.60 | 3011.17 | 23616.00 |

43

Fig. 1	Provinces  and Sub Basin in Yom River Basin

Fig. 2 Landuse and Inundate Risk  by agricultural land use/cover

Fig. 3  Annual Rainfall and Inundate Risk by rainfall

Fig. 4  Inundate Risk  by Slope and Soil Texture

Fig. 5   Inundate Risk  by density of water bodies and  Size of Sub Basins

Fig. 6  Inundate Risk  in Yom River Basin

Acknowledgement

LANDSAT Application and Classification Disaster Area on Yom River Basin project could not success

if the Regional Geo-Informatics and Space Technology, Lower Northern Region, Naresuan University was not received the very strongly support from Geo-Informatics and Space Technology Development Agency (Public Organization) or GISTDA in term of finance and satellite data. In this occasion, the authors would like to express

the great gratitude to all GISTDA administrative committee and the president of Naresuan University.

Reference

- Chen, G.Y. Chen. 1986. Chronological table of natural Disasters and Man-made Calamities in Various Dynasties of China. Shanghai Publishing House, June 1986.

- ESCAP/UNDP, Proceedings of the Expert Group Meeting on Comprehensive Flood Loss Prevention and Management, Bangkok 17–21 October 1988, United Nations, New York, 1988.

- Jeffrey A. Henderson, Robert W. Molacek, E.I.T., Charles F. Buzzard, and David K. Carlton, P.E. GIS AND THE FLOODPLAIN MANAGEMENT PLANNING PROCESS IN WASHINGTON STATE. Available at : http://gis.esri.com/library/userconf/proc96/ TO150/PAP149/ P149.HTM

- Yan, S.Y., S.X. Wang, C.J. Wei and F. Xu. 1988. Technical Report: Network-based Flood Quick Reporting System Using Remote Sensing, IRSA/CAS and SIC, November 1998.

- Yan, S.Y., C.J. Wei, S.X. Wang, and F. Xu. 1988. Multi-Technical Integrated System for Flood Monitoring in China. In Geoinformatics and Socioinformatics. The Proceedings of Geoinformatic’99 Conference, Li, B., et al., (eds). Amm Arbor, 19-21 June 1999, pp 1-9